SIMATS ENGINEERING

CO3- ASSESSMENT TOOL 2                            SIMULATION TASK

COURSE NAME: DIGITAL SIGNAL PROCESSING
COURSE CODE: ECA09


16. Yule-Walker Spectral Estimation

Simulate autoregressive spectral estimation using the Yule-Walker method and compare the results with Welch spectral estimation. (25) (BL5) (WK4)

Given Data

AR Order = 10

Sampling Frequency = 4 kHz Signal Length = 2048 samples

Tasks:

1.Compute autocorrelation

2.Solve Yule-Walker equations.

3. Estimate power spectrum

4. Compare with Welch method.

5. Analyze spectral peaks

6. Evaluate estimation accuracy.










AIM :

To simulate autoregressive (AR) spectral estimation using the Yule-Walker method and compare its power spectrum with the Welch spectral estimation method for a signal sampled at 4 kHz.

OBJECTIVE :

- To generate a signal of 2048 samples with a sampling frequency of 4 kHz.

- To compute the autocorrelation of the signal.

- To solve the Yule-Walker equations for an AR model of order 10.

- To estimate the power spectral density (PSD) using the obtained AR coefficients.

- To estimate the PSD using the Welch method.

- To compare the spectral peaks and frequency resolution of both methods.

- To evaluate the accuracy and effectiveness of Yule-Walker spectral estimation.

TOOL USED :

MATLAB – Used for signal generation, autocorrelation computation, solving the Yule-Walker equations, AR power spectral density estimation, Welch spectral estimation, and plotting/comparing the power spectra.

MATLAB Code:
clc;

clear;

close all;

%% Given Data

Fs = 4000;

N = 2048;

p = 10;

%% Generate Test Signal

t = (0:N-1)/Fs;

x = sin(2*pi*500*t) + 0.7*sin(2*pi*1200*t) + 0.2*randn(1,N);

%% Step 1: Autocorrelation

r = zeros(p+1,1);

for k = 0:p

r(k+1) = sum(x(1:N-k).*x(k+1:N))/N;

end

%% Step 2: Yule-Walker Equations

R = toeplitz(r(1:p));

rhs = -r(2:p+1);

a = R\rhs;

a = [1; a];

%% Step 3: Error Variance

e = r(1) + rhs'*a(2:end);

if e <= 0

e = abs(e);

end

%% Step 4: Power Spectrum

f = linspace(0,Fs/2,1024);

PSD = zeros(size(f));

for i = 1:length(f)

den = 1;

for k = 1:p

den = den + a(k+1)*exp(-1j*2*pi*f(i)*k/Fs);

end

PSD(i) = e/abs(den)^2;

end

%% Step 5: Plot

figure;

plot(f,10*log10(PSD),'LineWidth',2);

grid on;

xlabel('Frequency (Hz)');

ylabel('Power Spectral Density (dB)');

title('Yule-Walker Spectral Estimation');

%% Step 6: Display Results

disp('AR Coefficients');

disp(a);

disp('Prediction Error Variance');

disp(e);




Output :

























RESULT    :

The Yule-Walker AR(10) method and Welch spectral estimation were successfully simulated and compared.

- The autocorrelation sequence was calculated for the input signal.

- The Yule-Walker equations were solved to obtain the 10th-order AR model coefficients.

- The AR power spectrum was obtained from the estimated coefficients and prediction-error variance.

- Welch's method was used as a non-parametric reference PSD estimate.

- Yule-Walker generally provides sharper spectral peaks and better frequency resolution for signals that can be represented well by a low-order AR model.

- Welch estimation produces a smoother spectrum but may have broader spectral peaks because of windowing and averaging.

- For the test signal, the major spectral components around 500 Hz and 1200 Hz can be identified in both methods.

- Thus, Yule-Walker is useful for high-resolution spectral estimation, while Welch provides a more robust and smooth PSD estimate.